# Q. Demonstrate the RIP Routing in Packet Tracer.

Aim : To write a code to demonstrate RIP routing in packet tracer. Steps:

- In packet tracer take 2 PCs and 2 Routers.

- Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fasethernet0/0 using cross-copper cord.

- Connect both router via fastethernet0/1 using cross-copper cord.

- Set the IP address and default gateway of both the PCs in ip configuration of both the PCs respectively.

Topology:

Router0:

Router> enable

Router# configure terminal Router(config)# interface fastethernet0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1 Router(config-if)# ip address 10.0.0.1 255.255.255.0 Router(config-if)# no shutdown

Router(config-if)# exit Router(config)# router rip Router(config-router)# version 2

Router(config-router)# network 192.168.1.0

Router(config-router)# network 10.0.0.0 Router(config-router)# no auto-summary Router(config-router)# exit

Router(config)# end Router1:

Router> enable

Router# configure terminal

Router(config)# interface gigabitEthernet0/0 Router(config-if)# ip address 10.0.0.2 255.255.255.0 Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1 Router(config-if)# ip address 192.168.2.1 255.255.255.0 Router(config-if)# no shutdown

Router(config-if)# exit Router(config)# router rip Router(config-router)# version 2

Router(config-router)# network 10.0.0.0

Router(config-router)# network 192.168.2.0 Router(config-router)# no auto-summary Router(config-router)# exit

Router(config)# end Router#show ip route Router#show ip protocol Output:

Verification:In PC0 command prompt. C:>ipconfig

C:\>ping 192.168.2.2

Output:

Conclusion:The abovee topology has been executed successfully.