# Q. Demonstrate Routing OSPF in packet tracer.

Aim: To write a code to demonstrate routing OSPF in packet tracer. Steps:

- In packet tracer take 2 PCs and 2 Routers.

- Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fasethernet0/0 using cross-copper cord.

- Connect both router via fastethernet0/1 using cross-copper cord.

- Set the IP address and default gateway of both the PCs in ip configuration of both the PCs respectively.

Topology:

Code:

Router0:

Router> enable

Router# configure terminal Router(config)# interface fastethernet0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0 Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface fastethernet0/1 Router(config-if)# ip address 10.0.0.1 255.255.255.0 Router(config-if)# no shutdown

Router(config-if)# exit Router(config)#router ospf 1

Router(config-router)#network 192.168.1.0 0.0.0.255 area 0 Router(config-router)#exit

Router1:

Router> enable

Router# configure terminal Router(config)# interface fastethernet0/0

Router(config-if)# ip address 10.0.0.2 255.255.255.0 Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface fastethernet0/1

Router(config-if)# ip address 192.168.2.1 255.255.255.0 Router(config-if)# no shutdown

Router(config-if)# exit Router(config)#router ospf 2

Router(config-router)#network 10.0.0.0 0.0.0.255 area 0

Router(config-router)#network 192.168.2.0 0.0.0.255 area 0 Router(config-router)#exit

Verification :In PC in command Prompt.

PC0:

C:\ipconfig

C:\ping 192.168.2.10

Output:

Verification: On any one Router Router#show ip ospf neighbor Router#show ip protocol Router#show ip protocol Output:

Conclusion : The above Topology has been executed successfully.